Copyright © 2003 Agilent Technologies Inc. All rights reserved.

You have a royalty-free right to use, modify, reproduce and distribute the Sample Application Files (and/or any modified version) in any way you find useful, provided that you agree that Agilent has no warranty, obligations or liability for any Sample Application Files.

Agilent Technologies provides programming examples for illustration only.  Any sample program assumes that you are familiar with the programming language being demonstrated and the tools used to create and debug procedures. Agilent support engineers can help explain the

functionality of Agilent software components and associated commands, but they will not modify these samples to provide added functionality or construct procedures to meet your specific needs.

AGILENT 33220A 20 MHZ FUNCTION/ARBITRARY WAVEFORM GENERATOR

PROGRAMMING EXAMPLES

Organization

These files have been downloaded from www.agilent.com and are a copy of what is included on the CD for the Microsoft Visual Basic 6.0 development environment.

This document may refer to additional items included on the CD for other development environments.

The programming examples are organized by environment then by IO.  The topics of the example programs have been repeated within each folder to illustrate the same functionality with a different environment and/or IO (see Overview of Example Programs).  The programming environments include:

- Microsoft Visual Basic 6.0

- Miscrosoft Visual C 6.0

- ANSI-C (These examples will also work in VC 6.0)

- National Instruments LabView

- Agilent Technologies VEE

- Microsoft Excel 97 (limited examples)

The IO includes:

- Agilent VISA-COM

- IVI-COM

- VISA

- VXIplug&play

- ISDK

- National Instruments' NI488 IO Library (provided by National Instruments)

Programs listed in the Agilent Technologies 33220A User's Guide demonstrate Microsoft Visual Basic 6.0 using Agilent VISA-COM drivers and can be found in the folder \examples\chapter6\  These same programs are also located in VB6.0\VISAcom\

Select combinations of environments and IO have been demonstrated.  With the exception of the ISDK, you will need to acquire the environment and the IO independent of this CD.

- IDSK drivers are installed as a part of the free IntuiLink for Arb software included on this CD or may be obtained at http://www.agilent.com/find/intuilink

- VISA-COM, IVI-COM, VISA, and VXIplug&play drivers for the Agilent 33220A may be obtained at http://www.agilent.com/find/33220A in the Library under the heading Software, Firmware, & Drivers

- The NI488 IO Library is provided by National Instruments

Purpose

The example programs are provided solely for the purpose of illustrating a subset of the SCPI (Standard Commands for Programmable Instruments) commands used with the 33220A.  They are not to demonstrate the environment or the IO.

Overview of Example Programs

There are nine (9) example programs. The topics of the example programs have been repeated to illustrate the same functionality with a different environment and/or IO.

- SimpleSine: A basic program that selects the waveshape and sets its frequency, amplitude, and offset.

- Haversine: Uses burst to trigger sinewaves with a phase offset of 270 degrees.  Trigger source for this example is the bus.

- AMLowLevel: Configures a waveform with amplitude modulation using lower-level SCPI commands.  Also shows how to use the *SAV command to store the instrument configuration in memory.

- ASCIIarb: Downloads an arbitrary waveform to the generator as ASCII data.  Data values range from –1 to +1.  For faster downloads, use BinaryArb.

- BinaryArb: Downloads an arbitrary waveform to the generator as binary data.  Values range from –2047 to +2047.  For ASCII data, use ASCIIArb.

- LinearSweep: Creates a linear sweep for a sinusoid.  Sets start and stop frequency, as well as sweep time.

- Pulse: Configures a pulse waveshape with pulse width, period, and high/low levels.  The edge time is incremented.

- SquareBurst: Modifies the parameters for a square wave, including duty cycle, and outputs a set of three cycles upon receipt of each trigger.  Also shows how to enable and disable the SYNC output.

- PulseWidth: Configures a pulse with duty cycle which is then slowly modulated by a triangle waveform .

There are two example program s for Excel 97 or higher using VISA-COM, ISDK, or NI488 as the IO.  These examples download a column of data to the instrument as ASCII or binary data.

Using These Examples with Other Instruments

The example programs were created for the Agilent 33220A.  With slight modifications, the programs can also be used with the Agilent 33120A or Agilent 33250A.

33120A:

- Remove OUTPUT ON from all example programs; 33120A output is always on

- Pulse example will not work; 33120A does not have pulse capability

- PulseWidthMod example will not work; 33120A does not have pulse capability

- Examples using the VXIplug&play driver will not work as written

- Examples using the IVICom driver will not work as written. Replace the line Fgen.Initialize txtIO.Text, True, True with Fgen.Initialize txtIO.Text, False, True

33250A:

- PulseWidthMod example will not work; 33250A does not have PWM capability

- Examples using the IVICom driver will not work as written.  Replace the line Fgen.Initialize txtIO.Text, True, True with Fgen.Initialize txtIO.Text, False, True or include the 33250 driver